Panchip Microelectronics Co., Ltd. PAN2025 Peripheral User Guide Version: 1.0 Release date: Feb. 2020 Shanghai Panchip Microelectronics Co., Ltd. Address: Room 802, No. 666 summer Shanghai Zhangjiang Hi-Tech Park Road, People's Republic of China Tel: 021-50802371 Website: http://www.panchip.com

USING THIS DOCUMENT

The information in this document is subject to change without notice. Every effort has been made in the preparation of this document to ensure accuracy of the contents, but all statements, information, and recommendations in this document do not constitute a warranty of any kind, express or implied.

TRADEMARKS

Other names mentioned in this document are trademarks/registered trademarks of their respective owners.

DISCLAIMER

All or part of the products, services and features described in this document may not be within the purchase scope or the usage scope. Unless otherwise specified in the contract, all statements, information, and recommendations in this document are provided "AS IS" without warranties, guarantees or representations of any kind, either express or implied.

REVISION HISTORY

| Version | Date | Content | Reference |
| --- | --- | --- | --- |
| 1.0 | 02/09/2020 | Initial | - |

Contents

1 Analog to Digital Converter (ADC)	1

1.1 ADC Summary	1

1.2 ADC Register Structure	1

1.3 ADC Function API	1

1.3.1 ADC_TRGDLY	1

1.3.2 ADC_Enable	2

1.3.3 ADC_Disable	2

1.3.4 ADC_OPEN	2

1.3.5 ADC_Close	3

1.3.6 ADC_EnableHWTrigger	3

1.3.7 ADC_DisableHWTrigger	4

1.3.8 ADC_SetExtraSampleTime	4

1.3.9 ADC_SelInputRange	5

1.3.10 ADC_EnableInt	5

1.3.11 ADC_DisableInt	6

1.3.12 ADC_SeqModeEnable	6

1.3.13 ADC_SeqModeTriggerSrc	7

1.4 ADC Macro API	7

1.5 Notice	8

2 Flash Memory Controller (FMC)	9

2.1 FMC Summary	9

2.2 FMC Register Structure	9

2.3 FMC Function API	9

2.3.1 FMC_Open	9

2.3.2 FMC_Close	10

2.3.3 FMC_ReadCID	10

2.3.4 FMC_ReadPID	11

2.3.5 FMC_ReadUID	11

2.3.6 FMC_ReadUCID	12

2.3.7 FMC_page_erase	12

2.3.8 FMC_Write	12

2.3.9 FMC_Read	13

2.3.10 FMC_SetVectorPageAddr	13

2.3.11 FMC_GetVectorPageAddr	14

2.3.12 FMC_ReadDataFlashBaseAddr	14

2.3.13 FMC_GetCRC32Sum	15

2.4 Notice	15

3 General Purpose IO Interface (GPIO)	17

3.1 GPIO Summary	17

3.2 GPIO Register Structure	17

3.3 GPIO Function API	18

3.3.1 GPIO_SetMode	18

3.3.2 GPIO_EnableInt	18

3.3.3 GPIO_DisableInt	19

3.4 GPIO Macro API	19

3.5 Notice	20

4 Inter IC Bus Interface (I2C)	21

4.1 I2C Summary	21

4.2 I2C Register Structure	21

4.3 I2C Function API	21

4.3.1 I2C_Open	21

4.3.2 I2C_Close	22

4.3.3 I2C_ClearTimeoutFlag	22

4.3.4 I2C_Trigger	23

4.3.5 I2C_DisableInt	23

4.3.6 I2C_EnableInt	23

4.3.7 I2C_GetBusClockFreq	24

4.3.8 I2C_SetBusClockFreq	24

4.3.9 I2C_GetIntFlag	25

4.3.10 I2C_GetStatus	25

4.3.11 I2C_GetData	25

4.3.12 I2C_SetData	26

4.3.13 I2C_SetSlaveAddr	26

4.3.14 I2C_SetSlaveAddrMask	27

4.3.15 I2C_EnableTimeout	27

4.3.16 I2C_DisableTimeout	27

4.3.17 I2C_EnableWakeup	28

4.3.18 I2C_DisableWakeup	28

4.4 I2C Macro API	29

4.5 Notice	29

5 Pulse Width Modulation Interface (PWM)	30

5.1 PWM Summary	30

5.2 PWM Register Structure	30

5.3 PWM Function API	31

5.3.1 PWM_ConfigOutputChannel	31

5.3.2 PWM_EnableOutput	32

5.3.3 PWM_DisableOutput	32

5.3.4 PWM_Start	33

5.3.5 PWM_Stop	33

5.3.6 PWM_ForceStop	33

5.3.7 PWM_EnableZeroInt	34

5.3.8 PWM_DisableZeroInt	34

5.3.9 PWM_ClearZeroIntFlag	35

5.3.10 PWM_GetZeroIntFlag	35

5.3.11 PWM_EnableCenterInt	35

5.3.12 PWM_DisableCenterInt	36

5.3.13 PWM_ClearCenterIntFlag	36

5.3.14 PWM_GetCenterIntFlag	37

5.3.15 PWM_EnableCMPDInt	37

5.3.16 PWM_DisableCMPDInt	37

5.3.17 PWM_ClearCMPDIntFlag	38

5.3.18 PWM_GetCMPDIntFlag	38

5.3.19 PWM_EnableDeadZone	39

5.3.20 PWM_DisableDeadZone	39

5.3.21 PWM_EnableCMPUInt	40

5.3.22 PWM_DisableCMPUInt	40

5.3.23 PWM_ClearCMPUIntFlag	40

5.3.24 PWM_GetCMPUIntFlag	41

5.4 PWM Macro API	41

5.5 Notice	42

6 Serial Peripheral Interface (SPI)	43

6.1 SPI Summary	43

6.2 SPI Register Structure	43

6.3 SPI Function API	43

6.3.1 SPI_Open	43

6.3.2 SPI_Close	44

6.3.3 SPI_DisableAutoSS	44

6.3.4 SPI_EnableAutoSS	45

6.3.5 SPI_SetBusClock	45

6.3.6 SPI_GetBusClock	46

6.3.7 SPI_EnableInt	46

6.3.8 SPI_DisableInt	47

6.4 Notice	48

7 Timer (TMR)	49

7.1 TMR Summary	49

7.2 TMR Register Structure	49

7.3 TMR Function API	49

7.3.1 TIMER_Open	49

7.3.2 TIMER_Close	50

7.3.3 TIMER_Delay	50

7.4 TMR Macro API	51

8 Universal Asynchronous Receiver/Transmitter (UART)	53

8.1 UART Summary	53

8.2 UART Register Structure	53

8.3 UART Function API	53

8.3.1 UART_ClearIntFlag	53

8.3.2 UART_Close	54

8.3.3 UART_DisableInt	54

8.3.4 UART_EnableInt	55

8.3.5 UART_Open	56

8.3.6 UART_Read	56

8.3.7 UART_SetLine_Config	57

8.3.8 UART_SetTimeoutCnt	57

8.3.9 UART_Write	58

8.4 Notice	58

9 WatchDog Timer (WDT)	59

9.1 WDT Summary	59

9.2 WDT Register Structure	59

9.3 WDT Function API	59

9.3.1 WDT_Open	59

9.3.2 WDT_Close	60

9.3.3 WDT_EnableInt	60

9.3.4 WDT_DisableInt	61

9.3.5 WDT_SelectClockSource	61

9.4 WDT Macro API	62

10 Window WatchDog Timer (WWDT)	63

10.1 WWDT Summary	63

10.2 WWDT Register Structure	63

10.3 WWDT Function API	63

10.3.1 WWDT_Open	63

10.3.2 WWDT_Close	64

10.3.3 WWDT_SelectClockSource	64

10.4 WWDT Macro API	65

11 Contact Information	66

Analog to Digital Converter (ADC)

ADC Summary

PAN2025 contains a 12-bit 8-channel successive approximation analog-to-digital converter (SAR A/D), which could be triggered by software, external pin (STADC/P3.2) or PWM.

ADC Register Structure

typedef struct{

__I  uint32_t DAT;

__I  uint32_t RESERVED0[7];

__IO uint32_t CTL;

__IO uint32_t CHEN;

__IO uint32_t CMP0;

__IO uint32_t CMP1;

__IO uint32_t STATUS;

__I  uint32_t RESERVED1[4];

__IO uint32_t TRGDLY;

__IO uint32_t EXTSMPT;

__IO uint32_t SEQCTL;

__I  uint32_t SEQDAT1;

__I  uint32_t SEQDAT2;

__IO uint32_t CTL2;

}ADC_T;

ADC Function API

ADC_TRGDLY

Syntax

ADC_TRGDLY(ADC_T *adc, uint32_t u32Data)

Purpose

Delay the convert starting time after PWM triggered.

Parameters

ADC_T *adc：ADC structure，Base address of ADC module

uint32_t u32Data：Delay time

Return Value

None

ADC_Enable

Syntax

ADC_Enable(ADC_T *adc)

Purpose

Enable ADC conversion.

Parameters

ADC_T *adc：ADC structure，Base address of ADC module

Return Value

None

ADC_Disable

Syntax

ADC_Disable(ADC_T *adc)

Purpose

Disable ADC conversion.

Parameters

ADC_T *adc：ADC structure，Base address of ADC module

Return Value

None

ADC_OPEN

Syntax

ADC_Open(ADC_T *adc, uint32_t u32InputMode, uint32_t u32OpMode, uint32_t u32ChMask)

Purpose

Open specific ADC channel.

Parameters

ADC_T *adc：ADC structure，Base address of ADC module

uint32_t u32InputMode：NULL, not used

uint32_t u32OpMode：NULL, not used

uint32_t u32ChMask：Channel selection bit. BIT0 corresponds to ch0，BIT1 corresponds to ch1…

Return Value

None

ADC_Close

Syntax

ADC_Close(ADC_T *adc)

Purpose

Close (Disable) ADC.

Parameters

ADC_T *adc：ADC structure，Base address of ADC module

Return Value

None

ADC_EnableHWTrigger

Syntax

ADC_EnableHWTrigger(ADC_T *adc, uint32_t u32Source, uint32_t u32Param)

Purpose

Enable external hardware trigger.

Parameters

ADC_T *adc：ADC structure，Base address of ADC module

uint32_t u32Source：external trigger source, can be IO pin or PWM

uint32_t u32Param：If PWM trigger is selected, this parameter is used to set delay time between PWM and ADC conversion. Otherwise if external pin trigger is selected, this parameter is used to set trigger condition such as rising edge or falling edge

Return Value

None

ADC_DisableHWTrigger

Syntax

ADC_DisableHWTrigger(ADC_T *adc)

Purpose

Disable external hardware trigger.

Parameters

ADC_T *adc：ADC structure，Base address of ADC module

Return Value

None

ADC_SetExtraSampleTime

Syntax

ADC_SetExtraSampleTime(ADC_T *adc, uint32_t u32ChNum, uint32_t u32SampleTime)

Purpose

Set sampling time of specific ADC channel.

Parameters

ADC_T *adc：ADC structure，Base address of ADC module

uint32_t u32ChNum：NULL, not used

uint32_t u32SampleTime：ADC sampling time, can be 0/1/2/4... times of clock period.

Return Value

None

ADC_SelInputRange

Syntax

ADC_SelInputRange(uint32_t u32EnableHigh)

Purpose

Set range of input signal.

Parameters

uint32_t u32EnableHigh：Can be 0 or 1. If set to 1, input range can be 0.4V~2.4V (High Level Gear), otherwise if set to 0,input range can be 0.4V~1.4V (Low Level Gear).

Return Value

None

ADC_EnableInt

Syntax

ADC_EnableInt(ADC_T *adc, uint32_t u32Mask)

Purpose

Enable ADC interrupt.

Parameters

ADC_T *adc：ADC structure，Base address of ADC module

uint32_t u32Mask：Bit mask to select which interrupt to enable

Return Value

None

ADC_DisableInt

Syntax

ADC_DisableInt(ADC_T *adc, uint32_t u32Mask)

Purpose

Disable ADC interrupt.

Parameters

ADC_T *adc：ADC structure，Base address of ADC module

uint32_t u32Mask：Bit mask to select which interrupt to disable

Return Value

None

ADC_SeqModeEnable

Syntax

ADC_SeqModeEnable(ADC_T *adc, uint32_t u32SeqTYPE, uint32_t u32ModeSel)

Purpose

Enable ADC PWM sequential mode.

Parameters

ADC_T *adc：ADC structure，Base address of ADC module

uint32_t u32SeqTYPE：Indicate sequential sampling type.

uint32_t u32ModeSel：Determine which two channels to use, can be three modes: channel 01, channel 12, channel 02.

Return Value

None

ADC_SeqModeTriggerSrc

Syntax

ADC_SeqModeTriggerSrc(ADC_T *adc,

uint32_t u32SeqModeTriSrc1,

uint32_t u32SeqModeTriSrc2)

Purpose

Set trigger source in ADC PWM sequential mode.

Parameters

ADC_T *adc：ADC structure，Base address of ADC module

uint32_t u32SeqModeTriSrc1：The 1st PWM trigger source and type

uint32_t u32SeqModeTriSrc2：The 2nd PWM trigger source and type

Return Value

None

  - ADC Macro API

ADC_CONFIG_CH8(adc, u32Source)：Configure the analog input source of channel 8

ADC_GET_CONVERSION_DATA(adc, u32ChNum)：Get ADC conversion result data

ADC_GET_INT_FLAG(adc, u32Mask)：Get specific ADC interrupt flag

ADC_CLR_INT_FLAG(adc, u32Mask)：Clear specific ADC interrupt flag

ADC_IS_BUSY(adc)：Check if ADC is busy or not

ADC_IS_DATA_OVERRUN(adc, u32ChNum)：Check if ADC conversion data is overrun

ADC_IS_DATA_VALID(adc, u32ChNum)：Check if ADC conversion result data is valid or not

ADC_POWER_DOWN(adc)：ADC power down mode

ADC_SEQUENTIAL_DISABLE(adc)：Disable ADC sequential mode

ADC_TRG1CTL_TRIGGER(adc)：Select TRG1CTL as single interrupt sequential mode

ADC_TRG2CTL_TRIGGER(adc)：Select TRG1CTL as single interrupt sequential mode

ADC_ENABLE_CMP0(adc, u32ChNum, u32Condition, u32Data, u32MatchCount)：Enable CMP0

ADC_DISABLE_CMP0(adc)：Disable CMP0

ADC_ENABLE_CMP1(adc, u32ChNum, u32Condition, u32Data, u32MatchCount)：Enable CMP1

ADC_DISABLE_CMP1(adc)：Disable CMP1

ADC_SET_INPUT_CHANNEL(adc, u32Mask)：Set ADC input channel

ADC_START_CONV(adc)：Start A/D Conversion

ADC_STOP_CONV(adc)：Stop A/D Conversion

ADC_ENABLE_TESTMODE(adc)：Enable ADC test mode

ADC_DISABLE_TESTMODE(adc)：Disable ADC test mode

ADC_SET_CLKDIV(adc,div)：Set ADC click division

ADC_SET_CMPCTL(adc,current)：Set ADC compare current

  - Notice

- The resolution of ADC is 12Bit but the precision is only 10Bit, so the lowest two bits can be discarded in real application scenarios.

2. ADC would be calibrated before leave factory, and user should firstly read out the calibration data in INFO area in flash before using ADC.

- Flash Memory Controller (FMC)

  - FMC Summary

PAN2025 contains a 31KBytes on-chip flash, which can store user application code and data. The application code area (APPROM) can be up to 29KB, and the loader code area (LDROM) is 2KB. The data area can be flexibly configured and share memory with APROM. Additionally, In System Programming (ISP) and In Application Programming (IAP) is also supported on PAN2025.

  - FMC Register Structure

typedef struct{

__IO uint32_t ISPCTL;

__IO uint32_t ISPADR;

__IO uint32_t ISPDAT;

__IO uint32_t RESERVED0[3];

__IO uint32_t ISPCMD;

__IO uint32_t ISPTRG;

__O  uint32_t DFBA;

__IO uint32_t FATCTL;

__IO uint32_t DLYCTL;

__IO uint32_t RESERVED1[5];

__IO uint32_t ISPSTS;

__IO uint32_t RESERVED2[3];

__O  uint32_t ISPCFG0;

__O  uint32_t ISPCFG1;

__O  uint32_t ISPCFG2;

__O  uint32_t ISPCFG3;

}FMC_T;

  - FMC Function API

FMC_Open

Syntax

void FMC_Open(void);

Purpose

Enable ISP function of FMC

Parameters

None

Return Value

None

FMC_Close

Syntax

void FMC_Close(void);

Purpose

Disable ISP function of FMC

Parameters

None

Return Value

None

FMC_ReadCID

Syntax

uint32_t FMC_ReadCID(void);

Purpose

Read Company ID of flash

Parameters

None

Return Value

Company ID

FMC_ReadPID

Syntax

uint32_t FMC_ReadPID(void);

Purpose

Read Product ID of flash

Parameters

None

Return Value

Product ID

FMC_ReadUID

Syntax

uint32_t FMC_ReadUID(uint32_t index);

Purpose

Read Unique ID of flash

Parameters

None

Return Value

UID

FMC_ReadUCID

Syntax

uint32_t FMC_ReadUCID(uint32_t index);

Purpose

Read Unique Company ID of flash

Parameters

None

Return Value

UCID

FMC_page_erase

Syntax

int32_t FMC_page_erase(uint32_t u32PageAddr);

Purpose

Erase a flash page. (Page size is 512 bytes)

Parameters

u32PageAddr：Page address, should be aligned to 512 bytes

Return Value

None

FMC_Write

Syntax

void FMC_Write(uint32_t u32Addr, uint32_t *u32Data);

Purpose

Write 16 bytes to flash

Parameters

u32Addr：Flash address to write data in

u32Data：Data buffer which stores data to be written, should be 16 bytes align

Return Value

None

FMC_Read

Syntax

uint32_t FMC_Read(uint32_t u32Addr, uint32_t *pdata);

Purpose

Read 16 bytes from flash to the buffer pdata points to

Parameters

u32Addr：Flash address where read data from

pdata：Points to the buffer stores the read-out data

Return Value

0

FMC_SetVectorPageAddr

Syntax

void FMC_SetVectorPageAddr(uint32_t u32PageAddr);

Purpose

Set system memory vector address

Parameters

u32PageAddr：Address for jumping, generally can be address of APROM or LDROM

Return Value

None

FMC_GetVectorPageAddr

Syntax

uint32_t FMC_GetVectorPageAddr(void);

Purpose

Get system memory vector address

Parameters

None

Return Value

Vector address

FMC_ReadDataFlashBaseAddr

Syntax

uint32_t FMC_ReadDataFlashBaseAddr(void);

Purpose

Get the base address of data flash

Parameters

None

Return Value

Base address of Data flash

FMC_GetCRC32Sum

Syntax

int32_t FMC_GetCRC32Sum(uint32_t addr, uint32_t count, uint32_t *chksum);

Purpose

Calculate CRC32 of specific data on flash

Parameters

Addr：Start address of data on flash

Count：Bytes counter of data, should be 16 bytes aligned

Chksum：Calculating result

Return Value

0

Notice

- If there is only one application, the programming area can be up to 31KB.

- If APROM and LDROM are both used, please insure the configuration in Keil, that is, the R/O offset address of APROM is 0x00, the R/O offset address of LDROM is 0x100000.

- In applications that uses APROM/LDROM, Panlink Programmer would automatically set the CONFIG3 according to the size of APROM and LDROM, to insure the binary file not exceed one of the following specific size:

- APROM<29K，LDROM<2K.

- APROM<2K，LDROM<29k.

- Note of APAEND_DIS(CONFIG3[16]):

- APAEND_DIS=1

- APROM: 0x00~0x7c00,

- LDROM:0x100000~0x100800

- APAEND_DIS=0

- APROM:0x00~0x800

- LDROM:0x100000~0x107c00

- Unlock function should be called before operate FMC: SYS_UnlockReg()

- Note that a flash page should be erased before write.

- General Purpose IO Interface (GPIO)

  - GPIO Summary

There are up to 24 GPIO pins in PAN2025, and these pins can be configured to other special functions. All pins are assigned to 6 specific ports (P0/P1/P2/P3/P4/P5). Every pin is independent, and all the pins have individual registers to control the different functions and data.

The I/O type of each pin can be configured by software individually as Input, Push-pull output, Open-drain output, or Quasi-bidirectional mode. After the chip is reset, the I/O mode of all pins is stay in input mode and each port data register Px_DOUT[n] resets to 1. For Quasi-bidirectional mode, each I/O pin is equipped with a very weak individual pull-up resistor about 110k~ 300kΩ for VDD is from 2.2 V to 3.0 V.

Key features:

- Four I/O modes:

  - Quasi-bidirectional mode

  - Push-pull output

  - Open-drain output

  - Input-only with high impendence

- Quasi-bidirectional TTL/Schmitt trigger input mode selected by SYS_Px_MFP[23:16]

- I/O pin configured as interrupt source with edge/level setting

- I/O pin internal pull-up resistor enabled only in Quasi-bidirectional I/O mode

- Enabling the pin interrupt function will also enable the pin wake-up function

- High driver and high sink I/O mode support

  - GPIO Register Structure

typedef struct {

__IO uint32_t MODE;

__IO uint32_t DINOFF;

__IO uint32_t DOUT;

__IO uint32_t DATMSK;

__IO uint32_t PIN;

__IO uint32_t DBEN;

__IO uint32_t INTTYPE;

__IO uint32_t INTEN;

__IO uint32_t INTSRC;

}GPIO_T;

  - GPIO Function API

GPIO_SetMode

Syntax

GPIO_SetMode(GPIO_T *gpio, uint32_t u32PinMask, uint32_t u32Mode)

Purpose

Set GPIO input/output mode, including high-impedance input, push-pull output, open-drain output and quasi-bidirectional mode

parameters

GPIO_T *gpio：GPIO structure, base address of GPIO port, can be P0 to P5

uint32_t u32PinMask：Pin mask of specific GPIO port, value 0~7 corresponds to pin1~pin7

uint32_t u32Mode：GPIO operation mode, including high-impedance input, push-pull output, open-drain output and quasi-bidirectional mode

Return Value

None

GPIO_EnableInt

Syntax

GPIO_EnableInt(GPIO_T *gpio, uint32_t u32Pin, uint32_t u32IntAttribs)

Purpose

Enable interrupt of specific GPIO pin.

parameters

GPIO_T *gpio：GPIO structure, base address of GPIO port, can be P0 to P5

uint32_t u32Pin：GPIO pin number, can be 0~7

uint32_t u32IntAttribs：indicate interrupt trigger condition, can be high-level trigger, low-level trigger, rising-edge trigger, falling-edge trigger and both-edge trigger

Return Value

None

GPIO_DisableInt

Syntax

void GPIO_DisableInt(GPIO_T *gpio, uint32_t u32Pin)

Purpose

Disable interrupt of specific GPIO pin.

parameters

GPIO_T *gpio：GPIO structure, base address of GPIO port, can be P0 to P5

uint32_t u32Pin：GPIO pin number, can be 0~7

Return Value

None

  - GPIO Macro API

GPIO_CLR_INT_FLAG(gpio, u32PinMask)：Clear interrupt flag of specific GPIO pin

GPIO_CLR_ALLINT_FLAG(gpio)：Clear all interrupt flags of specific GPIO port

GPIO_DISABLE_DEBOUNCE(gpio, u32PinMask)：Disable debounce function of specific GPIO pin

GPIO_ENABLE_DEBOUNCE(gpio, u32PinMask)：Enable debounce function of specific GPIO pin

GPIO_DISABLE_DIGITAL_PATH(gpio, u32PinMask)：Disable digital input path of specific GPIO pin

GPIO_ENABLE_DIGITAL_PATH(gpio, u32PinMask)：Enable digital input path of specific GPIO pin

GPIO_DISABLE_PULLUP_PATH(gpio, u32PinMask)：Disable pull up path of specific GPIO pin

GPIO_ENABLE_PULLUP_PATH(gpio, u32PinMask)：Enable pull up path of specific GPIO pin

GPIO_DISABLE_DOUT_MASK(gpio, u32PinMask)：Disable data-out of specific GPIO pin

GPIO_ENABLE_DOUT_MASK(gpio, u32PinMask)：Enable data-out of specific GPIO pin

GPIO_GET_INT_FLAG(gpio, u32PinMask)：Get interrupt flag of specific GPIO pin

GPIO_SET_DEBOUNCE_TIME(clksrc, clksel)：Set debounce time

GPIO_GET_IN_DATA(gpio)：Get value of input-data register

GPIO_SET_OUT_DATA(gpio, data)：Set value of output-data register

GPIO_TOGGLE(u32Pin)：Toggle level of specific GPIO pin

GPIO_EnableEINT0：Enable GPIO external interrupt 0

GPIO_DisableEINT0：Enable GPIO external interrupt 1

Notice

- Pull-up resistor is needed if a GPIO pin work in open-drain mode

- In quasi-bidirectional mode, DOUT(Px_Dout[n]) of specific pin should be set to 1 to enable digital input

- GPIO have internal pull-up resistor which only works in quasi-bidirectional mode

- A 3V-synchronization operation should be done after set P56 mode if it works as GPIO

- Inter IC Bus Interface (I2C)

  - I2C Summary

PAN2025 contains an I2C controller. I2C is a two-wire, bi-directional serial bus that provides a simple and efficient method for data exchange between devices. The I2C standard is a true multi-master bus including collision detection and arbitration that prevents data corruption if two or more masters attempt to control the bus sim-ultaneously. There is one I2C0 controller which supports Power-down wake-up function.

  - I2C Register Structure

typedef struct{

__IO uint32_t CTL;

__IO uint32_t ADDR0;

__IO uint32_t DAT;

__I  uint32_t STATUS;

__IO uint32_t CLKDIV;

__IO uint32_t TOCTL;

__IO uint32_t ADDR1;

__IO uint32_t ADDR2;

__IO uint32_t ADDR3;

__IO uint32_t ADDRMSK0;

__IO uint32_t ADDRMSK1;

__IO uint32_t ADDRMSK2;

__IO uint32_t ADDRMSK3;

__I  uint32_t RESERVED0[2];

__IO uint32_t CTL1;

__IO uint32_t STATUS1;

}I2C_T;

  - I2C Function API

I2C_Open

Syntax

uint32_t I2C_Open(I2C_T *i2c, uint32_t u32BusClock)

Purpose

Make I2C module be ready and set the wanted bus clock.

parameters

I2C_T *i2c：I2C structure, base address of I2C module

u32BusClock：The target bus speed of I2C module

Return Value

Actual I2C bus clock frequency

I2C_Close

Syntax

void I2C_Close(I2C_T *i2c)

Purpose

Close the I2C module

parameters

I2C_T *i2c：I2C structure, base address of I2C module

Return Value

None

I2C_ClearTimeoutFlag

Syntax

void I2C_ClearTimeoutFlag(I2C_T *i2c)

Purpose

Clears the timeout flag

parameters

I2C_T *i2c：I2C structure, base address of I2C module

Return Value

None

I2C_Trigger

Syntax

void I2C_Trigger(I2C_T *i2c, uint8_t u8Start, uint8_t u8Stop, uint8_t u8Si, uint8_t u8Ack)

Purpose

Sets the control bit of the I2C module

parameters

I2C_T *i2c：I2C structure, base address of I2C module

u8Start：Sets START bit to I2C module

u8Stop：Sets STOP bit to I2C module

u8Si：Sets SI bit to I2C module

u8Ack：Sets ACK bit to I2C module

Return Value

None

I2C_DisableInt

Syntax

void I2C_DisableInt(I2C_T *i2c)

Purpose

Disables the interrupt (EI bit) of I2C module

parameters

I2C_T *i2c：I2C structure, base address of I2C module

Return Value

None

I2C_EnableInt

Syntax

void I2C_EnableInt(I2C_T *i2c)

Purpose

Enables the interrupt (EI bit) of I2C module

parameters

I2C_T *i2c：I2C structure, base address of I2C module

Return Value

None

I2C_GetBusClockFreq

Syntax

uint32_t I2C_GetBusClockFreq(I2C_T *i2c)

Purpose

Gets the real bus clock of I2C module

parameters

I2C_T *i2c：I2C structure, base address of I2C module

Return Value

Actual I2C bus clock frequency

I2C_SetBusClockFreq

Syntax

uint32_t I2C_SetBusClockFreq(I2C_T *i2c, uint32_t u32BusClock)

Purpose

Sets the real bus clock of I2C module

parameters

I2C_T *i2c：I2C structure, base address of I2C module

u32BusClock：The target bus speed of I2C module

Return Value

Actual I2C bus clock frequency

I2C_GetIntFlag

Syntax

uint32_t I2C_GetIntFlag(I2C_T *i2c)

Purpose

Gets the interrupt flag (SI bit) of I2C module

parameters

I2C_T *i2c：I2C structure, base address of I2C module

Return Value

Interrupt flag

I2C_GetStatus

Syntax

uint32_t I2C_GetStatus(I2C_T *i2c)

Purpose

Get the status of I2C module

parameters

I2C_T *i2c：I2C structure, base address of I2C module

Return Value

I2C status

I2C_GetData

Syntax

uint32_t I2C_GetData(I2C_T *i2c)

Purpose

Gets the data stored in data register of I2C module

parameters

I2C_T *i2c：I2C structure, base address of I2C module

Return Value

I2C data

I2C_SetData

Syntax

void I2C_SetData(I2C_T *i2c, uint8_t u8Data)

Purpose

Writes the data to data register of I2C module

parameters

I2C_T *i2c：I2C structure, base address of I2C module

u8Data：The data which will be write to data register of I2C module

Return Value

None

I2C_SetSlaveAddr

Syntax

void I2C_SetSlaveAddr(I2C_T *i2c, uint8_t u8SlaveNo, uint8_t u8SlaveAddr, uint8_t u8GCMode)

Purpose

Configure slave address and enable GC mode.

parameters

I2C_T *i2c：I2C structure, base address of I2C module

u8SlaveNo：The set number of salve address

u8SlaveAddr：The slave address

u8GCMode：enable GC mode

Return Value

None

I2C_SetSlaveAddrMask

Syntax

void I2C_SetSlaveAddrMask(I2C_T *i2c, uint8_t u8SlaveNo, uint8_t u8SlaveAddrMask)

Purpose

Configure the mask of slave address. The corresponding address bit is "Don't Care"

parameters

I2C_T *i2c：I2C structure, base address of I2C module

u8SlaveNo：The set number of salve address

u8SlaveAddrMask：The slave address mask

Return Value

None

I2C_EnableTimeout

Syntax

void I2C_EnableTimeout(I2C_T *i2c, uint8_t u8LongTimeout)

Purpose

Enables timeout function and configures DIV4 function to support long timeout.

parameters

I2C_T *i2c：I2C structure, base address of I2C module

u8LongTimeout：Enable timeout counter input clock is divide by 4

Return Value

None

I2C_DisableTimeout

Syntax

void I2C_DisableTimeout(I2C_T *i2c)

Purpose

Disables timeout function

parameters

I2C_T *i2c：I2C structure, base address of I2C module

Return Value

None

I2C_EnableWakeup

Syntax

uint32_t I2C_Open(I2C_T *i2c, uint32_t u32BusClock)

Purpose

Enables the wakeup function of I2C module

parameters

I2C_T *i2c：I2C structure, base address of I2C module

Return Value

None

I2C_DisableWakeup

Syntax

void I2C_DisableWakeup(I2C_T *i2c)

Purpose

Disables the wakeup function of I2C module

parameters

I2C_T *i2c：I2C structure, base address of I2C module

Return Value

None

  - I2C Macro API

I2C_SET_CONTROL_REG(i2c, u8Ctrl)：Sets the I2C control register at one time

I2C_WAIT_READY(i2c)：Wait for I2C module ready

I2C_DISABLE_CLOCK_STRETCH(i2c)：Disables clock stretch function

I2C_ENABLE_CLOCK_STRETCH(i2c)：Enables clock stretch function

I2C_GET_DATA(i2c)：Get the data stored in data register of I2C module

I2C_SET_DATA(i2c, u8Data)：Writes the data to data register of I2C module

I2C_GET_STATUS(i2c)：Get the status of I2C module

I2C_GET_TIMEOUT_FLAG(i2c)：Get the timeout flag of I2C module

I2C_GET_WAKEUP_FLAG(i2c)：Get the wakeup flag of I2C module

I2C_CLEAR_WAKEUP_FLAG(i2c)：Clears wakeup flag of I2C module

  - Notice

A module reset operation should be done after configure GPIO pins as I2C function:

SYS_ResetModule(I2CX_RST, 1000000);

- Pulse Width Modulation Interface (PWM)

  - PWM Summary

PAN2025 contains one PWM unit which is specially designed for motor driving control applications. The PWM0 supports eight PWM generators which can be configured as eight independent PWM outputs, PWM0_CH0~PWM0_CH7, or as four complementary PWM pairs, (PWM0_CH0, PWM0_CH1), (PWM0_CH2, PWM0_CH3), (PWM0_CH4, PWM0_CH5) and (PWM0_CH6, PWM0_CH7) with four programmable dead-time generators, or as four synchronous PWM pairs, with each pin in a pair in-phase (PWM0_CH0, PWM0_CH1), (PWM0_CH2, PWM0_CH3), (PWM0_CH4, PWM0_CH5) and (PWM0_CH6, PWM0_CH7).

Every complementary PWM pairs share one 8-bit prescaler. There are eight clock dividers providing five divided frequencies (1, 1/2, 1/4, 1/8, 1/16) for each channel. Each PWM output has independent 16-bit counter for PWM period control, and 16-bit comparators for PWM duty control. The eight PWM generators provide sixteen independent PWM interrupt flags which are set by hardware when the corresponding PWM period counter comparison matched period and duty. Each PWM interrupt source with its corresponding enable bit can request PWM interrupt. The PWM generators can be configured as Auto-reload mode to output PWM waveform continuously.

To prevent PWM driving output pin with unsteady waveform, the 16-bit period down counter and 16-bit comparator are implemented with double buffer. When user writes data to counter/comparator buffer registers, the updated value will be loaded into the 16-bit down counter/comparator at the end of current period. The double buffering feature avoids glitch at PWM outputs.

Besides PWM, Motor controlling also need Timer and ADC to work together. In order to control motor more precisely, we provide some registers that not only configure PWM but also Timer and ADC, by doing so, it can save more CPU time and control motor with ease especially in BLDC.

  - PWM Register Structure

typedef struct{

__IO uint32_t CLKPSC;

__IO uint32_t CLKDIV;

__IO uint32_t CTL;

__IO uint32_t PERIOD0;

__IO uint32_t PERIOD1;

__IO uint32_t PERIOD2;

__IO uint32_t PERIOD3;

__IO uint32_t PERIOD4;

__IO uint32_t PERIOD5;

__IO uint32_t PERIOD6;

__IO uint32_t PERIOD7;

__IO uint32_t CMPDAT0;

__IO uint32_t CMPDAT1;

__IO uint32_t CMPDAT2;

__IO uint32_t CMPDAT3;

__IO uint32_t CMPDAT4;

__IO uint32_t CMPDAT5;

__IO uint32_t CMPDAT6;

__IO uint32_t CMPDAT7;

__IO uint32_t CTL2;

__IO uint32_t FLAG;

__IO uint32_t INTEN;

__IO uint32_t INTSTS;

__IO uint32_t POEN;

__I  uint32_t RESERVED0[1];

__IO uint32_t DTCTL;

__IO uint32_t ADCTCTL0;

__IO uint32_t ADCTCTL1;

__IO uint32_t ADCTSTS0;

__IO uint32_t ADCTSTS1;

__I  uint32_t RESERVED1[4];

__IO uint32_t PCACTL;

}PWM_T;

  - PWM Function API

PWM_ConfigOutputChannel

Syntax

uint32_t PWM_ConfigOutputChannel (PWM_T *pwm,uint32_t u32ChannelNum,uint32_t u32Frequency,uint32_t u32DutyCycle,uint32_t u8OperateType)

Purpose

Configure parameters of specific output channel.

parameters

PWM_T *pwm：PWM structure, base address of PWM module

uint32_t u32ChannelNum：PWM channel number, can be 0~7

uint32_t u32Frequency：PWM output frequency

uint32_t u32DutyCycle：PWM output duty cycle ratio

uint32_t u8OperateType：PWM operation type, including edge-counting, center-counting and precise center-counting mode

Return Value

Expected waveform frequency

PWM_EnableOutput

Syntax

void PWM_EnableOutput (PWM_T *pwm, uint32_t u32ChannelMask)

Purpose

Enable output of specific PWM channel

parameters

PWM_T *pwm：PWM structure, base address of PWM module

uint32_t u32ChannelMask：PWM channel mask, can be 0~7

Return Value

None

PWM_DisableOutput

Syntax

void PWM_DisableOutput (PWM_T *pwm, uint32_t u32ChannelMask)

Purpose

Enable output of specific PWM channel

parameters

PWM_T *pwm：PWM structure, base address of PWM module

uint32_t u32ChannelMask：PWM channel mask, can be 0~7

Return Value

None

PWM_Start

Syntax

void PWM_Start(PWM_T *pwm, uint32_t u32ChannelMask)

Purpose

Start PWM counting

parameters

PWM_T *pwm：PWM structure, base address of PWM module

uint32_t u32ChannelMask：PWM channel mask, can be 0~7

Return Value

None

PWM_Stop

Syntax

void PWM_ Stop (PWM_T *pwm, uint32_t u32ChannelMask)

Purpose

Stop PWM waveform and keep output low level in the next periods.

parameters

PWM_T *pwm：PWM structure, base address of PWM module

uint32_t u32ChannelMask：PWM channel mask, can be 0~7

Return Value

None

PWM_ForceStop

Syntax

void PWM_ForceStop (PWM_T *pwm, uint32_t u32ChannelMask)

Purpose

Stop PWM waveform immediately

parameters

PWM_T *pwm：PWM structure, base address of PWM module

uint32_t u32ChannelMask：PWM channel mask, can be 0~7

Return Value

None

PWM_EnableZeroInt

Syntax

void PWM_ EnableZeroInt (PWM_T *pwm, uint32_t u32ChannelNum)

Purpose

Enable PWM zero point interrupt

parameters

PWM_T *pwm：PWM structure, base address of PWM module

uint32_t u32ChannelMask：PWM channel mask, can be 0~7

Return Value

None

PWM_DisableZeroInt

Syntax

void PWM_ DisableZeroInt (PWM_T *pwm, uint32_t u32ChannelNum)

Purpose

Disable PWM zero point interrupt

parameters

PWM_T *pwm：PWM structure, base address of PWM module

uint32_t u32ChannelMask：PWM channel mask, can be 0~7

Return Value

None

PWM_ClearZeroIntFlag

Syntax

void PWM_ ClearZeroIntFlag (PWM_T *pwm, uint32_t u32ChannelNum)

Purpose

Clear flag of PWM zero point interrupt

parameters

PWM_T *pwm：PWM structure, base address of PWM module

uint32_t u32ChannelMask：PWM channel mask, can be 0~7

Return Value

None

PWM_GetZeroIntFlag

Syntax

uint32_t PWM_GetZeroIntFlag (PWM_T *pwm, uint32_t u32ChannelNum)

Purpose

Get flag of PWM zero point interrupt

parameters

PWM_T *pwm：PWM structure, base address of PWM module

uint32_t u32ChannelMask：PWM channel mask, can be 0~7

Return Value

Zero point interrupt flag

PWM_EnableCenterInt

Syntax

void PWM_EnableCenterInt (PWM_T *pwm, uint32_t u32ChannelNum,  uint32_t u32IntPeriodType)

Purpose

Enable PWM center point interrupt

parameters

PWM_T *pwm：PWM structure, base address of PWM module

uint32_t u32ChannelMask：PWM channel mask, can be 0~7

uint32_t u32IntPeriodType：Interrupt period type.

Return Value

None

PWM_DisableCenterInt

Syntax

void PWM_DisableCenterInt (PWM_T *pwm, uint32_t u32ChannelNum)

Purpose

Disable PWM center point interrupt

parameters

PWM_T *pwm：PWM structure, base address of PWM module

uint32_t u32ChannelMask：PWM channel mask, can be 0~7

Return Value

None

PWM_ClearCenterIntFlag

Syntax

void PWM_ClearCenterIntFlag (PWM_T *pwm, uint32_t u32ChannelNum)

Purpose

Clear flag of PWM center point interrupt

parameters

PWM_T *pwm：PWM structure, base address of PWM module

uint32_t u32ChannelMask：PWM channel mask, can be 0~7

Return Value

None

PWM_GetCenterIntFlag

Syntax

uint32_t PWM_GetCenterIntFlag (PWM_T *pwm, uint32_t u32ChannelNum)

Purpose

Get flag of PWM center point interrupt

parameters

PWM_T *pwm：PWM structure, base address of PWM module

uint32_t u32ChannelMask：PWM channel mask, can be 0~7

Return Value

Flag of center point interrupt

PWM_EnableCMPDInt

Syntax

void PWM_EnableCMPDInt (PWM_T *pwm, uint32_t u32ChannelNum)

Purpose

Enable PWM compare-down interrupt

parameters

PWM_T *pwm：PWM structure, base address of PWM module

uint32_t u32ChannelMask：PWM channel mask, can be 0~7

Return Value

None

PWM_DisableCMPDInt

Syntax

void PWM_DisableCMPDInt (PWM_T *pwm, uint32_t u32ChannelNum)

Purpose

Disable PWM compare-down interrupt

parameters

PWM_T *pwm：PWM structure, base address of PWM module

uint32_t u32ChannelMask：PWM channel mask, can be 0~7

Return Value

None

PWM_ClearCMPDIntFlag

Syntax

void PWM_ClearCMPDIntFlag (PWM_T *pwm, uint32_t u32ChannelNum)

Purpose

Clear flag of PWM compare-down interrupt

parameters

PWM_T *pwm：PWM structure, base address of PWM module

uint32_t u32ChannelMask：PWM channel mask, can be 0~7

Return Value

None

PWM_GetCMPDIntFlag

Syntax

uint32_t PWM_GetCMPDIntFlag (PWM_T *pwm, uint32_t u32ChannelNum)

Purpose

Get flag of PWM compare-down interrupt

parameters

PWM_T *pwm：PWM structure, base address of PWM module

uint32_t u32ChannelMask：PWM channel mask, can be 0~7

Return Value

Flag of compare-down interrupt

PWM_EnableDeadZone

Syntax

void PWM_EnableDeadZone (PWM_T *pwm, uint32_t u32ChannelNum, uint32_t u32Duration)

Purpose

Enable PWM dead zone.

parameters

PWM_T *pwm：PWM structure, base address of PWM module

uint32_t u32ChannelMask：PWM channel mask, can be 0~7

uint32_t u32Duration：Dead-zone time, = PWM_CLK * (DTInm+1). (nm here can be 01, 23, 45)

Return Value

None

PWM_DisableDeadZone

Syntax

void PWM_DisableDeadZone (PWM_T *pwm, uint32_t u32ChannelNum)

Purpose

Disable PWM dead zone

parameters

PWM_T *pwm：PWM structure, base address of PWM module

uint32_t u32ChannelMask：PWM channel mask, can be 0~7

Return Value

None

PWM_EnableCMPUInt

Syntax

void PWM_EnableCMPUInt (PWM_T *pwm, uint32_t u32ChannelNum)

Purpose

Enable PWM compare-up interrupt

parameters

PWM_T *pwm：PWM structure, base address of PWM module

uint32_t u32ChannelMask：PWM channel mask, can be 0~7

Return Value

None

PWM_DisableCMPUInt

Syntax

void PWM_DisableCMPUInt (PWM_T *pwm, uint32_t u32ChannelNum)

Purpose

Disable PWM compare-up interrupt

parameters

PWM_T *pwm：PWM structure, base address of PWM module

uint32_t u32ChannelMask：PWM channel mask, can be 0~7

Return Value

None

PWM_ClearCMPUIntFlag

Syntax

void PWM_ClearCMPUIntFlag (PWM_T *pwm, uint32_t u32ChannelNum)

Purpose

Clear flag of PWM compare-up interrupt

parameters

PWM_T *pwm：PWM structure, base address of PWM module

uint32_t u32ChannelMask：PWM channel mask, can be 0~7

Return Value

None

PWM_GetCMPUIntFlag

Syntax

uint32_t PWM_GetCMPUIntFlag (PWM_T *pwm, uint32_t u32ChannelNum)

Purpose

Get flag of PWM compare-up interrupt

parameters

PWM_T *pwm：PWM structure, base address of PWM module

uint32_t u32ChannelMask：PWM channel mask, can be 0~7

Return Value

Flag of compare-up interrupt

  - PWM Macro API

PWM_ENABLE_INDEPENDENT_MODE(pwm)：Enable independent mode

PWM_ENABLE_COMPLEMENTARY_MODE(pwm)：Enable complementary mode

PWM_DISABLE_COMPLEMENTARY_MODE(pwm)：Disable complementary mode

PWM_ENABLE_GROUP_MODE(pwm)：Enable group mode

PWM_DISABLE_GROUP_MODE(pwm)：Disable groupt mode

PWM_ENABLE_SYNC_MODE(pwm)：Enable synchronization mode

PWM_DISABLE_SYNC_MODE(pwm)：Disable synchronization mode

PWM_ENABLE_OUTPUT_INVERTER(pwm, u32ChannelMask)：Invert output

PWM_SET_PRESCALER(pwm, u32ChannelNum, u32Prescaler)：Set precaler of specific channel

PWM_SET_DIVIDER(pwm, u32ChannelNum, u32Divider)：set division factor of specific channel

PWM_SET_CMR(pwm, u32ChannelNum, u32CMR)：Set duty cycle of specific channel, and the corresponding formula is:

Edge Mode: DutyCycle = (CMPn+1)/(PERIODn+1).

Center-align Mode: DutyCycle = (PERIODn - CMPn)/(PERIODn+1).

Precise Center-align Mode: DutyCycle = (PERIOD–(CMP+1)*2) / PERIOD

PWM_SET_CNR(pwm, u32ChannelNum, u32CNR)：Set period value of specific channel

PWM_SET_CMRD(pwm, u32ChannelNum, u32CMRD)：Set compare-down reference value under asymmetric mode

PWM_SET_ALIGNED_TYPE(pwm, u32ChannelMask, u32AlignedType)：Set align type of specific channel

PWM_ENABLE_ASYMMETRIC_MODE(pwm)：Enable asymmetric mode

PWM_ENABLE_PCAEN(pwm)：Enable precise center-aligned mode

PWM_SET_CNTMODE(pwm,u32ChannelNum,u8CntMode)：Set counting mode of specific channel

  - Notice

- PWM counter module does not support one-shot mode

- Serial Peripheral Interface (SPI)

  - SPI Summary

PAN2025 contains two channels of Serial Peripheral Interface (SPI). The SPI applies to synchronous serial data communication and allows full duplex transfer. Devices communicate in Master/Slave mode with 4-wire bi-direction interface. The SPI controller performing a serial-to-parallel conversion on data received from a peripheral device, and a parallel-to-serial conversion on data transmitted to a peripheral device. SPI controller can be configured as a master or a slave device.

  - SPI Register Structure

typedef struct{

__IO uint32_t CTL;

__IO uint32_t CLKDIV;

__IO uint32_t SSCTL;

__I  uint32_t RESERVED0[1];

__I  uint32_t RX;

__I  uint32_t RESERVED1[3];

__O  uint32_t TX;

__I  uint32_t RESERVED2[6];

__IO uint32_t SLVCTL;

__IO uint32_t FIFOCTL;

__IO uint32_t STATUS;

}SPI_T;

  - SPI Function API

SPI_Open

Syntax

uint32_t SPI_Open(PAN_SPI_T *spi,

uint32_t u32MasterSlave,

uint32_t u32SPIMode,

uint32_t u32DataWidth,

uint32_t u32BusClock)

Purpose

SPI module initialization.

Parameters

PAN_SPI_T *spi：SPI structure, base address of SPI module

uint32_t u32MasterSlave：Set current SPI as master or slave, can be SPI_SLAVE or SPI_MASTER

uint32_t u32SPIMode：Set SPI mode, can be:

SPI_MODE_0		// CLKP=0; RX_NEG=0; TX_NEG=1

SPI_MODE_1		// CLKP=0; RX_NEG=1; TX_NEG=0

SPI_MODE_2		// CLKP=1; RX_NEG=1; TX_NEG=0

SPI_MODE_3		// CLKP=1; RX_NEG=0; TX_NEG=1

uint32_t u32DataWidth：Set SPI data width in bit, can be 8~32

uint32_t u32BusClock：Set SPI clock frequency

Return Value

Real SPI working clock frequency

SPI_Close

Syntax

void SPI_Close(PAN_SPI_T *spi)

Purpose

Disable SPI

Parameters

PAN_SPI_T *spi：SPI structure, base address of SPI module

Return Value

None

SPI_DisableAutoSS

Syntax

void SPI_DisableAutoSS(PAN_SPI_T *spi, uint32_t u32ActiveLevel)

Purpose

Disable auto slave selection

Parameters

PAN_SPI_T *spi：SPI structure, base address of SPI module

uint32_t u32ActiveLevel：Set active level, can be SPI_SS_ACTIVE_HIGH or SPI_SS_ACTIVE_LOW

Return Value

None

SPI_EnableAutoSS

Syntax

void SPI_EnableAutoSS(PAN_SPI_T *spi, uint32_t u32SSPinMask, uint32_t u32ActiveLevel)

Purpose

Disable auto slave selection

Parameters

PAN_SPI_T *spi：SPI structure, base address of SPI module

uint32_t u32SSPinMask：pin mask, can be SPI_SS

uint32_t u32ActiveLevel：Set active level, can be SPI_SS_ACTIVE_HIGH or SPI_SS_ACTIVE_LOW

Return Value

None

SPI_SetBusClock

Syntax

uint32_t SPI_SetBusClock(PAN_SPI_T *spi, uint32_t u32BusClock)

Purpose

Set SPI bus clock

Parameters

PAN_SPI_T *spi：SPI structure, base address of SPI module

uint32_t u32BusClock：Set SPI clock frequency

Return Value

Real SPI working clock frequency

SPI_GetBusClock

Syntax

uint32_t SPI_GetBusClock(PAN_SPI_T *spi)

Purpose

Get SPI bus clock

Parameters

PAN_SPI_T *spi：SPI structure, base address of SPI module

Return Value

SPI bus clock

SPI_EnableInt

Syntax

void SPI_EnableInt(PAN_SPI_T *spi, uint32_t u32Mask)

Purpose

Enalbe SPI interrupt

Parameters

PAN_SPI_T *spi：SPI structure, base address of SPI module

uint32_t u32Mask：Specific interrupt mask, can be:

SPI_IE_MASK						//Transmission complete

SPI_SSTA_SSINAIEN_MASK		//slave selecting fail

SPI_FIFO_RXOV_INTEN_MASK	//receive buffer overflow

Return Value

None

SPI_DisableInt

Syntax

void SPI_DisableInt(PAN_SPI_T *spi, uint32_t u32Mask)

Purpose

Disable SPI interrupt

Parameters

PAN_SPI_T *spi：SPI structure, base address of SPI module

uint32_t u32Mask：Specific interrupt mask, can be:

SPI_IE_MASK						//Transmission complete

SPI_SSTA_SSINAIEN_MASK		//slave selecting fail

SPI_FIFO_RXOV_INTEN_MASK	//receive buffer overflow

Return Value

None

  - Notice

UART0 clock should be enabled before using SPI

- Timer (TMR)

  - TMR Summary

PAN2025 contains three 32-bit timers, TMR0, TMR1 and TMR2, allowing user to easily implement a timer control for applications. The timer can perform functions, such as frequency measurement, delay timing, clock generation, event counting by external input pins, and interval measurement by external capture pins.

  - TMR Register Structure

typedef struct{

__IO uint32_t CTL;

__IO uint32_t CMP;

__IO uint32_t INTSTS;

__I  uint32_t CNT;

__I  uint32_t CAP;

__IO uint32_t EXTCTL;

__IO uint32_t EINTSTS;

}TIMER_T;

  - TMR Function API

TIMER_Open

Syntax

uint32_t TIMER_Open(TIMER_T *timer, uint32_t u32Mode, uint32_t u32Freq)

Purpose

Open timer and configure working mode and counting period.

parameters

TIMER_T *timer：Timer structure, base address of Timer module, can be TIMER0, TIMER1, TIMER2

uint32_t u32Mode：Timer counting mode, including one-shot mode, periodic mode, toggle output mode and continuous counting mode

uint32_t u32Freq：Expected working frequency

Return Value

Real working frequency

TIMER_Close

Syntax

void TIMER_Close(TIMER_T *timer)

Purpose

Stop counting and close interrupt.

parameters

TIMER_T *timer：Timer structure, base address of Timer module, can be TIMER0, TIMER1, TIMER2

Return Value

None

TIMER_Delay

Syntax

void TIMER_Delay(TIMER_T *timer, uint32_t u32Usec)

Purpose

Delay some time in micro second level.

parameters

TIMER_T *timer：Timer structure, base address of Timer module, can be TIMER0, TIMER1, TIMER2

uint32_t u32Usec：Delay time, range is 10us~1s

Return Value

None

  - TMR Macro API

TIMER_SET_CMP_VALUE(timer, u32Value)：Set comparator reference value

TIMER_SET_PRESCALE_VALUE(timer, u32Value)：Set precaler value

TIMER_IS_ACTIVE(timer)：Check if specific timer is active or not

TIMER_Start(TIMER_T *timer)：Start timer counting

TIMER_Stop(TIMER_T *timer)：Stop timer counting

TIMER_Reset(TIMER_T *timer)：Reset Timer counting, prescale counter and CNTEN

TIMER_EnableWakeup(TIMER_T *timer)：Enable timer wakeup function

TIMER_DisableWakeup(TIMER_T *timer)：Disable timer wakeup function

TIMER_EnableCaptureDebounce(TIMER_T *timer)：Enable capture debounce function

TIMER_DisableCaptureDebounce(TIMER_T *timer)：Disable capture debounce function

TIMER_EnableEventCounterDebounce(TIMER_T *timer)：Enable event counter debounce function

TIMER_DisableEventCounterDebounce(TIMER_T *timer)：Disable event counter debounce function

TIMER_EnableInt(TIMER_T *timer)：Enable timeout interrupt

TIMER_DisableInt(TIMER_T *timer)：Disable timeout interrupt

TIMER_EnableCaptureInt(TIMER_T *timer)：Enable capture interrupt

TIMER_DisableCaptureInt(TIMER_T *timer)：Disable capture interrupt

TIMER_GetIntFlag(TIMER_T *timer)：Get timeout interrupt flag

TIMER_ClearIntFlag(TIMER_T *timer)：Cleare timeout interrupt flag

TIMER_GetTFFlag(TIMER_T *timer)：Get timeout event flag

TIMER_ClearTFFlag(TIMER_T *timer)：Clear timeout event flag

TIMER_GetCaptureIntFlag(TIMER_T *timer)：Get capture interrupt flag

TIMER_ClearCaptureIntFlag(TIMER_T *timer)：Clear capture interrupt flag

TIMER_GetCaptureFlag(TIMER_T *timer)：Get capture event flag

TIMER_ClearCaptureFlag(TIMER_T *timer)：Clear capture event flag

TIMER_GetWakeupFlag(TIMER_T *timer)：Get wakup event flag

TIMER_ClearWakeupFlag(TIMER_T *timer)：Clear wakeup event flag

TIMER_SetCaptureSource(TIMER_T *timer,uint32_t Source)：Set capture source

TIMER_GetCaptureData(TIMER_T *timer)：Get capture data

TIMER_GetCounter(TIMER_T *timer)：Get current counter value

- Universal Asynchronous Receiver/Transmitter (UART)

  - UART Summary

PAN2025 contains two channels of Universal Asynchronous Receiver/Transmitters(UART). The UART0/1/2/3 performs a serial-to-parallel conversion on data received from the peripheral, and a parallel-to-serial conversion on data transmitted from the CPU. The UART0/1/2/3 channel supports five types of interrupts. The UART0/1/2/3 has 8 bytes Receiver/Transmitter FIFO.

  - UART Register Structure

typedef struct{

__IO uint32_t DAT;

__IO uint32_t INTEN;

__IO uint32_t FIFO;

__IO uint32_t LINE;

__I  uint32_t RESERVED0[2];

__IO uint32_t FIFOSTS;

__IO  uint32_t INTSTS;

__IO uint32_t TOUT;

__IO uint32_t BAUD;

}UART_T;

  - UART Function API

UART_ClearIntFlag

Syntax

void UART_ClearIntFlag(PAN_UART_T* uart , uint32_t u32InterruptFlag);

Purpose

Clear interrupt flag

Parameters

PAN_UART_T* uart：UART structure, base address of UART module, can be UART0 / UART1 / UART2 / UART3

uint32_t u32InterruptFlag：UART interrupt flag, can be:

UART_INTSTS_RLSINT_Msk				Receive Line Status Interrupt

UART_INTSTS_BUFERRINT_Msk			Buffer Error Interrupt

Return Value

None

UART_Close

Syntax

void UART_Close(PAN_UART_T* uart);

Purpose

Close UART

Parameters

PAN_UART_T* uart：UART structure, base address of UART module, can be UART0 / UART1 / UART2 / UART3

Return Value

None

UART_DisableInt

Syntax

void UART_DisableInt(PAN_UART_T*  uart, uint32_t u32InterruptFlag );

Purpose

Disable UART interrupt

Parameters

PAN_UART_T* uart：UART structure, base address of UART module, can be UART0 / UART1 / UART2 / UART3

uint32_t u32InterruptFlag：UART interrupt flag, can be:

UART_INTEN_RXTOIEN_Msk			Rx Time Out interrupt

UART_INTEN_BUFERRIEN_Msk		Buffer Error Interrup

UART_INTEN_RLSIEN_Msk			Rx Line status interrupt

UART_INTEN_THREIEN_Msk			Tx empty interrupt

UART_INTEN_RDAIEN_Msk			Rx ready interrupt

Return Value

None

UART_EnableInt

Syntax

void UART_EnableInt(PAN_UART_T*  uart, uint32_t u32InterruptFlag );

Purpose

Enable UART interrupt

Parameters

PAN_UART_T* uart：UART structure, base address of UART module, can be UART0 / UART1 / UART2 / UART3

uint32_t u32InterruptFlag：UART interrupt flag, can be:

UART_INTEN_RXTOIEN_Msk			Rx Time Out interrupt

UART_INTEN_BUFERRIEN_Msk		Buffer Error Interrup

UART_INTEN_RLSIEN_Msk			Rx Line status interrupt

UART_INTEN_THREIEN_Msk			Tx empty interrupt

UART_INTEN_RDAIEN_Msk			Rx ready interrupt

Return Value

None

UART_Open

Syntax

void UART_Open(PAN_UART_T* uart, uint32_t u32baudrate);

Purpose

Open specific UART and configure baudrate

Parameters

PAN_UART_T* uart：UART structure, base address of UART module, can be UART0 / UART1 / UART2 / UART3

uint32_t u32baudrate，UART baudrate, suggested values are 921600 / 460800 / 230400 / 115200 / 57600 / 38400 / 19200 / 9600 / 4800

Return Value

None

UART_Read

Syntax

uint32_t UART_Read(PAN_UART_T* uart, uint8_t *pu8RxBuf, uint32_t u32ReadBytes)

Purpose

Read data from UART Rx FIFO

Parameters

PAN_UART_T* uart：UART structure, base address of UART module, can be UART0 / UART1 / UART2 / UART3

uint8_t *pu8RxBuf：The buffer to receive the data of receive FIFO.

uint32_t u32ReadBytes：The the read bytes number of data.

Return Value

Receive byte count

UART_SetLine_Config

Syntax

void UART_SetLine_Config(PAN_UART_T* uart, uint32_t u32baudrate, uint32_t u32data_width, uint32_t u32parity, uint32_t  u32stop_bits)

Purpose

Configure UART parameters

Parameters

PAN_UART_T* uart：UART structure, base address of UART module, can be UART0 / UART1 / UART2 / UART3

uint32_t u32baudrate：UART baudrate, suggested values are 921600 / 460800 / 230400 / 115200 / 57600 / 38400 / 19200 / 9600 / 4800

uint32_t u32data_width：UART data width, can be: UART_WORD_LEN_5, UART_WORD_LEN_6, UART_WORD_LEN_7, UART_WORD_LEN_8

uint32_t u32parity：UART parity, can be: UART_PARITY_NONE, UART_PARITY_ODD, UART_PARITY_EVEN, UART_PARITY_MARK, UART_PARITY_SPACE

uint32_t u32stop_bits：UART stop bits, can be UART_STOP_BIT_1, UART_STOP_BIT_1_5, UART_STOP_BIT_2

Return Value

None

UART_SetTimeoutCnt

Syntax

void UART_SetTimeoutCnt(PAN_UART_T* uart, uint32_t u32TOC);

Purpose

Set Tx timeout count

Parameters

PAN_UART_T* uart：UART structure, base address of UART module, can be UART0 / UART1 / UART2 / UART3

uint32_t u32TOC： timeout count, can be 40~255

Return Value

None

UART_Write

Syntax

uint32_t UART_Write(PAN_UART_T* uart,uint8_t *pu8TxBuf, uint32_t u32WriteBytes);

Purpose

Write data to UART Tx FIFO

Parameters

PAN_UART_T* uart：UART structure, base address of UART module, can be UART0 / UART1 / UART2 / UART3

uint8_t *pu8RxBuf：The buffer to send the data to UART transmission FIFO.

uint32_t u32ReadBytes：The byte number of data.

Return Value

Transfer byte count

  - Notice

P56 pin can be configured as Rx of UART0, and note that 3V-synchronization operation should be done when using this pin（RCC_Wait3vSyncReady()）.

- WatchDog Timer (WDT)

  - WDT Summary

PAN2025 contains a watch dog timer, which is used to perform a system reset when system runs into an unknown state. This prevents system from hanging for an infinite period of time. Besides, the Watchdog Timer supports the function to wake-up system from Idle/Power-down mode.

  - WDT Register Structure

typedef struct{

__IO uint32_t CTL;

__IO uint32_t ALTCTL;

}WDT_T;

  - WDT Function API

WDT_Open

Syntax

void  WDT_Open(uint32_t u32TimeoutInterval, uint32_t u32ResetDelay, uint32_t u32EnableReset, uint32_t u32EnableWakeup)

Purpose

Configure WDT parameters and start counting

Parameters

uint32_t u32TimeoutInterval：WDT timeout interval, can be: WDT_TIMEOUT_2POW4 / WDT_TIMEOUT_2POW6 / WDT_TIMEOUT_2POW8 / WDT_TIMEOUT_2POW10 / WDT_TIMEOUT_2POW12 / WDT_TIMEOUT_2POW14 / WDT_TIMEOUT_2POW16 / WDT_TIMEOUT_2POW18

uint32_t u32ResetDelay：Delay time before reset system, can be: WDT_RESET_DELAY_2CLK / WDT_RESET_DELAY_17CLK / WDT_RESET_DELAY_129CLK / WDT_RESET_DELAY_1025CLK

uint32_t u32EnableReset：Enable WDT reset or not

uint32_t u32EnableWakeup：Enable WDT wakeup function or not

Return Value

None

WDT_Close

Syntax

void WDT_Close(void)

Purpose

Stop WDT counting

Parameters

None

Return Value

None

WDT_EnableInt

Syntax

void WDT_EnableInt(void)

Purpose

Enable WDT timeout interrupt

Parameters

None

Return Value

None

WDT_DisableInt

Syntax

void WDT_DisableInt(void)

Purpose

Disable WDT timeout interrupt

Parameters

None

Return Value

None

WDT_SelectClockSource

Syntax

void WDT_SelectClockSource(uint32_t u32clksel)

Purpose

Select WDT clock source

Parameters

uint32_t u32clksel：WDT clock source selection, can be: CLK_APB_WDTSEL_APB1_DIV2048 / CLK_APB_WDTSEL_LIRC

Return Value

None

  - WDT Macro API

WDT_CLEAR_RESET_FLAG()：Clear WDT reset flag

WDT_CLEAR_TIMEOUT_FLAG()：Clear WDT timeout flag

WDT_CLEAR_TIMEOUT_INT_FLAG()：Clear WDT timeout interrupt flag

WDT_CLEAR_TIMEOUT_WAKEUP_FLAG()：Clear WDT wakeup flag

WDT_GET_RESET_FLAG()：Get WDT reset flag

WDT_GET_TIMEOUT_FLAG()：Get WDT timeout flag

WDT_GET_TIMEOUT_INT_FLAG()：Get WDT timeout interrupt flag

WDT_GET_TIMEOUT_WAKEUP_FLAG()：Get WDT wakeup flag

WDT_RESET_COUNTER()：Reset the 18-bit WDT counter

- Window WatchDog Timer (WWDT)

  - WWDT Summary

PAN2025 contains a Window WatchDog Timer (WWDT), which is used to perform a system reset within a specified window period to prevent software run to uncontrollable status by any unpredictable condition.

  - WWDT Register Structure

typedef struct{

__O  uint32_t RLDCNT;

__IO uint32_t CTL;

__IO uint32_t STATUS;

__I  uint32_t CNT;

}WWDT_T;

  - WWDT Function API

WWDT_Open

Syntax

void WWDT_Open(uint32_t u32PreScale, uint32_t u32CmpValue, uint32_t u32EnableInt)

Purpose

Configure WWDT parameters and start counting

Parameters

uint32_t u32PreScale：WWDT prescale, can be:WWDT_PRESCALER_1 / WWDT_PRESCALER_2 / WWDT_PRESCALER_4 / WWDT_PRESCALER_8 / WWDT_PRESCALER_16 / WWDT_PRESCALER_32 / WWDT_PRESCALER_64 / WWDT_PRESCALER_128 / WWDT_PRESCALER_192 / WWDT_PRESCALER_256 / WWDT_PRESCALER_384 / WWDT_PRESCALER_512 / WWDT_PRESCALER_768 / WWDT_PRESCALER_1024 / WWDT_PRESCALER_1536 / WWDT_PRESCALER_2048

uint32_t u32CmpValue：Window compare reference value, can be 0x0~0x3f

uint32_t u32EnableInt：Enable interrupt or not

Return Value

None

WWDT_Close

Syntax

void WWDT_Close(void)

Purpose

Stop WWDT counting

Parameters

None

Return Value

None

WWDT_SelectClockSource

Syntax

void WWDT_SelectClockSource(uint32_t u32clksel)

Purpose

Select WWDT clock source

Parameters

uint32_t u32clksel：WWDT clock source selection, can be: CLK_APB_WWDTSEL_APB1_DIV204 / CLK_APB_WWDTSEL_LIRC

Return Value

None

  - WWDT Macro API

WWDT_CLEAR_RESET_FLAG()：Clear WWDT reset flag

WWDT_CLEAR_INT_FLAG()：Clear WWDT window match interrupt flag

WWDT_CLEAR_WWDTF_FLAG()：Clear WWDT window match event flag

WWDT_GET_RESET_FLAG()：Get WWDT reset flag

WWDT_GET_INT_FLAG()：Get WWDT window match interrupt flag

WWDT_GET_WWDTF_FLAG()：Get WWDT window match event flag

WWDT_GET_COUNTER()：Get WWDT counter

WWDT_RELOAD_COUNTER()：Reload WWDT counter to initial value 0x3F

Contact Information

上海磐启微电子有限公司
电话：021-50802371
传真：021-50802372
地址：中国（上海）自由贸易试验区盛夏路666号E栋802

苏州磐启微电子有限公司
电话：0512-68136052
传真：0512-68136051
地址：苏州工业园区崇文路199号富华科技大厦3-F

上海磐启微电子有限公司深圳分公司
电话：0755-26403799
传真：0755-26403799
地址：深圳市南山区科技园科技路11号伟杰大厦106室